Using a Static Library with Qt Creator on Raspberry Pi

1. Introduction

This document outlines the process of preparing a build setup, flashing a Raspberry Pi, testing, and integrating a static library with Qt Creator to build a user interface (UI) application. The static library will provide core functionalities, while Qt Creator will handle the UI, offering options based on the library.

This workflow is ideal for projects requiring a modular structure, such as integrating H.264 decoding capabilities with a custom UI on Raspberry Pi.

2. How to Prepare the Build Setup

Prerequisites:

- Raspberry Pi 4 with a Linux-based operating system.
- Cross-compilation toolchain (e.g., `aarch64-linux-gnu-gcc` for ARM architecture).
- Qt Creator installed on your development machine.
- Static library source code (e.g., `libh264dec.a`).

Steps:

1. Install Required Tools:
   ```bash
   sudo apt update
   sudo apt install build-essential gcc g++ make
   ```
2. Clone the Source Code:
   ```bash
   git clone <https://github.com/Ajith3204/H264_DECODER.git>
   cd <H264_DECODER>
   ```
3. Build the Static Library:
   - Ensure the `Makefile` is configured for cross-compilation if needed. For example:
     ```make
     CC = aarch64-linux-gnu-gcc
     ```
   - Run the following commands:
     ```bash
     make clean
     make all
     ```
   - Verify the static library (`libh264dec.a`) is generated.

3. Steps to Flash and Test

Flashing to Raspberry Pi:

1. Transfer the Library and Files:
   - Use `scp` to copy the static library and other necessary files to the Raspberry Pi:
     ```bash
     scp   libh264dec.a pi@<raspberry_pi_ip>:/home/pi/project/
     ```
2. Install Dependencies on Raspberry Pi:
   ```bash
   sudo apt install libqt5-dev qtcreator
   ```
3. Test the Static Library:
   - Write a small test program linking the static library. Example:
     ```c
     gcc -o test_h264 test_h264.c -L/home/pi/project/ -lh264dec -lpthread
     ./test_h264
     ```

Testing on Raspberry Pi:

Verify the output of the test program to ensure the static library functions as expected.

4. Source Code Location

The source code for the static library and the UI application is available on GitHub:

- Static Library Repository: [https://github.com/Ajith3204/H264_DECODER.git)
- UI Application Repository: [https://github.com/Ajith3204/h264_UI.git)

5. Transferring the Static Library to Qt Creator

Steps:

1. Set Up Qt Creator:
   - Open Qt Creator and create a new project or open an existing one.
   - Go to `Projects > Build & Run > Build Settings` and configure the Raspberry Pi Kit.
2. Add the Static Library:
   - Add the static library to your `.pro` file:
     ```
     LIBS += -L/home/pi/project/ -lh264dec
     INCLUDEPATH += /home/pi/project/
     ```
3. Use the Library in Your UI Code:
   - Include the relevant header files in your Qt application source files.
     ```cpp
     #include "h264dec.h"
     ```
   - Link the library functions to corresponding UI actions (e.g., buttons for Play, Pause).
4. Deploy and Run:
   - Use Qt Creator's deployment tools to transfer the application to the Raspberry Pi.
   - Test the UI to ensure it interacts correctly with the static library.