Week 1 – Orientation & Embedded Fundamentals
1. 	Branching Model (Simple Industry Model)

Main branches:

main        → production code
develop     → integration branch
feature/*   → new features
bugfix/*    → bug fixing

example

project/
│
├── components/
│   ├── gpio_driver/
│   ├── uart_driver/
│   └── can_driver/
│
├── main/
│   └── app_main.c
│
├── CMakeLists.txt
└── sdkconfig

2.	GPIO:

When the GPIO is

- Input mode try to set mode pullup or pull down,because it is will be in the high impedance state without that pin will be floating,directly read 0 or 1, which cause unstable behaviour and false triggering.
Recommended is PULLUP mode:
1. Prevent Floating 
2. Give stable default input
3. Avoid False Triggering
4. Improve Noise Immunity
5. Safer in case of short Circuit
6. Most MCU have internally pull up reg.
Interrupt mode then decide negative edge 
as Digital signal have two stages 0 or 1
low =0, high =3.3
Positive edge: low =high ,0 =3.3
Negative edge: high = low, 3.3 =0

| Configuration | Default state | Button Pressed State | Edge |
| --- | --- | --- | --- |
| Pull_up | High | Low | Negative/Falling Edge |
| Pull Down | Low | High | Positive /Raising edge |

Debouncing: When Mechanical button gets pressed that does not change state directly from Low to high, only once presses it changes its state continuously high-low-high -low.
To avoid: ignore button event for short time (20ms).

3.	Interrupt:
Polling method
 Wastes CPU
 Not scalable
 Bad for low-power & RTOS systems

Interrupt-based

CPU reacts only when button is pressed
Clean, efficient
Industry standard

What is a Framework?

A framework is basically a pre-written set of code and libraries that lets you easily interact with hardware without writing everything from scratch.

Without a framework, you’d have to manually configure every GPIO, timer, or peripheral using registers — which is possible but much harder.

4.	PWM:

Program Flow

1. 	PWM Timer Configuration

• 	You create a structure.

• 	Set parameters like:

• 	Mode: High-speed mode ()

• 	Timer number: 0

• 	Duty resolution: 8-bit (values 0–255)

• 	Frequency: 5 kHz

• 	Call  → this initializes the timer hardware.

2.	PWM Channel Configuration

You define a ledc_channel_config_t structure.

Assign:

GPIO pin = 2

Channel = 0

Timer = 0

Initial duty = 0

Call ledc_channel_config(&ledc_channel) → this links the timer to the GPIO pin, so PWM can drive the LED.

3.	Main Loop (LED fading)

Infinite loop (while(1)).

Fade up:

Duty cycle increases from 0 → 255.

Each step:

ledc_set_duty() stores the new duty value.

ledc_update_duty() immediately applies it to hardware.

vTaskDelay(10 ms) slows down the change so you can see the LED gradually brighten.

Fade down:

Duty cycle decreases from 255 → 0.

Same process, but LED gradually dims.

5.	Timers:

app_main()

↓

Configure GPIO2 as output

↓

Create FreeRTOS timer (1s period)

↓

Start timer

↓

FreeRTOS scheduler waits...s

↓

Every 1s → led_timer_callback()

↓

Toggle led_state

↓

Set GPIO2 HIGH/LOW

↓

LED blinks forever

6.	I2C : Inter integrated circuit
1. Initialize I²C

i2c_master_init() is called.

- Builds an i2c_config_t struct with:

  - Mode = Master

  - SDA pin = GPIO 21

  - SCL pin = GPIO 22

  - Internal pull-ups enabled

  - Clock speed = 100 kHz

- Applies configuration with i2c_param_config().

- Installs driver with i2c_driver_install().

👉 At this point, the ESP32’s I²C controller (I2C_NUM_0) is ready to talk to devices

3. Start Scanning

- Logs "Scanning I2C bus...".

4. Loop Through All Addresses

- For each possible 7-bit I²C address (1 → 126):

- Tries to send a write command to that address.

- If the device acknowledges (ACK), ret == ESP_OK.

- Logs "Device found at address: 0xXX".

I²C bus   The frequency (e.g., 100 kHz, 400 kHz) is the clock speed of the SCL line.

- It defines how fast bits are transferred across the bus.

- Every bit of data on SDA is synchronized with a clock pulse on SCL.

- So the frequency is essentially the “data rate” of communication.

lines (SDA = data, SCL = clock) are open-drain (or open-collector).

- That means devices on the bus can only pull the line low (connect it to ground).

- No device actively drives the line high. Instead, the line floats unless something pulls it up.

- To make sure the bus idles at a logical HIGH when no one is pulling it low, you need pull-up resistors.
Traditionally, you add external resistors (like 4.7kΩ to 10kΩ) from SDA/SCL to VCC (3.3V).

- ESP32 also allows you to enable internal pull-ups in software

This saves you from wiring external resistors in simple setups.

- However, internal pull-ups are relatively weak (~45kΩ).
For reliable communication, especially with multiple devices or higher speeds, external resistors are recommended.

What “open‑drain” means

• 	Each I²C pin (SDA, SCL) is implemented as a transistor that can only pull the line down to ground.

• 	When the device wants to send a logic 0, it closes the transistor → line goes LOW.

• 	When the device wants to send a logic 1, it simply releases the transistor → the pin is left “open” (high‑impedance).

• 	Because no one actively drives the line HIGH, the bus would float unless something else defines the HIGH level

Why it’s needed

1. 	Synchronization

• 	I²C is synchronous: the master generates a clock (SCL) so all devices know when to read/write bits.

• 	Without a defined frequency, devices wouldn’t know how fast to sample SDA.

2. 	Data Transfer Speed

• 	Higher frequency = faster communication.

• 	Example: At 100 kHz, each bit takes 10 µs. At 400 kHz, each bit takes 2.5 µs.

3. 	Device Compatibility

• 	Each I²C device has a maximum supported frequency (listed in its datasheet).

• 	If you exceed it, the device may miss bits or fail to ACK.

4. 	Electrical Stability

• 	Faster clocks require stronger pull-ups and cleaner wiring.

• 	Slower clocks are more tolerant of long wires, breadboards, and noisy environments.

Command Link Concept

- A command link is essentially a container (or list) that holds a sequence of I²C operations you want to perform.

- Instead of calling each operation directly, you build them into this link step by step.

- Once the link is complete, you hand it to the driver, which executes the entire sequence atomically (without interruption).

- I²C transactions often involve multiple steps:

  - START condition

  - Send address

  - Write data

  - Read data

  - STOP condition

- ESP-IDF lets you record all these steps into a command link first, then execute them in one go.
This ensures the hardware sees them as one continuous transaction.

7.	SPI :

Default State of CS

• 	CS is normally HIGH (inactive).

• 	When CS = HIGH, the slave device (RC522) ignores the SPI bus.

• 	When the master wants to talk to the slave:

1. Pull CS LOW → this selects the RC522.

2. Send clock pulses + data on MOSI.

3. Receive data on MISO.

4. Pull CS HIGH again → transaction ends.

1.System Initialization

  - Configure SPI bus (MISO, MOSI, SCLK, CS).

  - Add RC522 as an SPI device.

  - Reset RC522 and configure basic registers (Tx/Rx settings, antenna on).

- Idle Loop

  - Continuously poll for a card.

  - Send a “Request” command (REQA) to check if a card is present.

- Card Detection

  - If RC522 responds, a card is in range.

  - Run Anti-collision to get the card’s unique ID (UID).

- Card Selection

  - Send “Select” command with UID.

  - RC522 confirms card is selected.

- Data Handling

  - ESP32 now has the UID.

  - You can:

  - Print UID on LCD (I²C).

  - Send UID via SIM800L (UART).

  - Or just log it for now.

- Loop Back

- Go back to idle loop, waiting for next card.

Start

↓

Init SPI + RC522

↓

Configure RC522 (antenna on)

↓

Loop:

├─> Send REQA

│    ├─ No card → repeat

│    └─ Card found → Anti-collision

↓

Get UID

↓

Select card

↓

Use UID (print/log/send)

↓

Back to loop

8. UART:

UART: there are 3 uart

Complete UART Flow

Include headers

↓

Choose UART + pins

↓

uart_config_t

↓

uart_param_config()

↓

uart_set_pin()

↓

uart_driver_install()

↓

uart_write_bytes()

uart_read_bytes()

9.	WiFi

1.NVS init				//Wi-Fi driver stores calibration data in NVS, so you must init

↓

2. Initialize Networking & Event Loop	//These prepare the TCP/IP stack and event system

↓

3 Wi-Fi Driver Initialisaton

↓

4.Registering Event Handler	//This is where ESP32 learns how to “listen” for Wi-Fi and IP events after the driver is initialized.

↓

5.Configuring WiFi SSID and Password   //this is where you actually tell ESP32 which router to connect to.

What Event Group actually does

It stores status as bits

It wakes up tasks waiting for those bits
 It does NOT control Wi-Fi
 It does NOT create Wi-Fi events

Step 1. NVS Init

- NVS = Non-Volatile Storage.

- It’s a small key-value database stored in ESP32’s flash memory.

- “Non-volatile” means data stays even after reset or power off.

- ESP-IDF uses NVS internally to store system data like:

- Wi-Fi calibration values (RF tuning parameters)

- Saved credentials (if you enable Wi-Fi auto-connect)

- Other libraries (like BLE, MQTT configs) may also use it

Step 2 .Component

1. esp_netif_init()

- Initializes the TCP/IP stack (network interface system).

- Sets up internal structures so ESP32 can manage IP addresses, routing, and sockets.

- Without this, ESP32 cannot talk to the network at all.

2. esp_event_loop_create_default()

- Creates the default event loop.

- ESP-IDF is event-driven: Wi-Fi, IP, and MQTT drivers post events (like “connected,” “got IP,” “disconnected”).

- This loop listens for those events and dispatches them to your handlers.

- Without it, you wouldn’t be able to catch Wi-Fi or IP events.

3. esp_netif_create_default_wifi_sta()

- Creates a default Wi-Fi station (STA) interface.

- “Station mode” means ESP32 acts like a client device connecting to a router.

- This interface object (esp_netif_t *) holds IP info (IP, subnet, gateway, DNS).

- DHCP updates this object when ESP32 gets an IP.

- You can query it later with esp_netif_get_ip_info().

Step 3. WiFi Initialisation
  Takes a configuration struct (wifi_init_config_t cfg) — usually created with WIFI_INIT_CONFIG_DEFAULT().

- Initializes the Wi-Fi driver:

  - Allocates internal buffers (TX/RX).

  - Sets up FreeRTOS tasks and queues for Wi-Fi events.

  - Loads WPA/WPA2 security functions.

  - Links the driver with the network interface created earlier (esp_netif_create_default_wifi_sta()).

- Prepares the ESP32 to accept further commands like esp_wifi_set_mode(), esp_wifi_set_config(), and esp_wifi_start().

🔹 What Happens Internally

- Creates Wi-Fi driver tasks (for scanning, connecting, transmitting).

- Allocates RX/TX buffers in RAM.

- Registers event callbacks with the event loop.

- Reads calibration data from NVS (Step 1).

- Prepares hardware for actual use.

Step 4.  Registeriung Event Handler

ESP‑IDF is event-driven.

• 	Wi‑Fi driver posts events like started, connected, disconnected.

• 	TCP/IP stack posts events like got IP.

• 	You must register handlers so your app can react to these events (e.g., retry connection, start MQTT).

Step 5. Applying the Configuration

Once you’ve filled the struct, you pass it to the Wi-Fi driver

esp_wifi_set_mode(WIFI_MODE_STA) → tells ESP32 to act as a client device.

- esp_wifi_set_config(WIFI_IF_STA, &wifi_config) → loads your SSID/password into the driver.

🔹 Typical Flow After Config

- Set mode → WIFI_MODE_STA.

- Apply config → SSID + password.

- Start Wi-Fi → esp_wifi_start().

- Event handler catches → WIFI_EVENT_STA_START → esp_wifi_connect().

- Router authenticates → WIFI_EVENT_STA_CONNECTED.

- DHCP assigns IP → IP_EVENT_STA_GOT_IP.

wifi_config_t holds SSID + password.

- esp_wifi_set_mode() sets ESP32 as client.

- esp_wifi_set_config() applies your credentials.

- After this, ESP32 can start Wi-Fi and connect to your router.

Step 6.  Starting the WIFi
What esp_wifi_start() Does

- Activates the Wi-Fi driver you initialized in Step 3.

- Starts background FreeRTOS tasks for scanning, connecting, and handling packets.

- Triggers the first event: WIFI_EVENT_STA_START.

- At this point, ESP32 is ready to connect, but hasn’t yet joined the router — you must call esp_wifi_connect() (usually inside the event handler).

Step 7. Event Handler Logic — this is where ESP32 reacts to Wi-Fi and IP events, retries if disconnected, and where you can hook in MQTT or other network tasks once you’re online.

  Event handler is the brain of Wi-Fi flow.

- It ensures ESP32 connects, retries, and knows when it’s online.

- Once IP is obtained, you can launch higher-level protocols like MQTT.

10. HTTP

LED ON/Off using OTA 
Browser (phone / laptop)

|  HTTP request  ( /on or /off )

↓

ESP32 HTTP ServeR

|  URI handler function

↓

GPIO driver

↓

LED hardware (ACTIVE-LOW)

1.Browser Request (Client Side)

- A user on a phone or laptop opens a browser and types:

  - http:///on → request to turn LED ON

  - http:///off → request to turn LED OFF

- This is a standard HTTP GET request sent over Wi-Fi to the ESP32.

2. ESP32 HTTP Server

- The ESP32 runs a lightweight HTTP server (using esp_http_server library).

- It listens for incoming requests on port 80.

- When a request arrives, the server checks the URI path (/on or /off).

3. URI Handler Function

- You register handler functions for specific URIs:

  - /on → mapped to a function that sets GPIO low (LED ON, since active-low).

  - /off → mapped to a function that sets GPIO high (LED OFF).

- Inside the handler:

- Parse the request.

- Call the GPIO driver functions to change pin state.

- Optionally send back an HTTP response (like "LED is ON").

4. GPIO Driver

- The ESP-IDF GPIO driver provides functions like:

  - gpio_set_level(pin, value) → sets pin HIGH or LOW.

- Since your LED is active-low:

- gpio_set_level(LED_PIN, 0) → LED ON

- gpio_set_level(LED_PIN, 1) → LED OFF

🔌 Why Active-Low Is Common

- Historical TTL Logic (Transistor-Transistor Logic)

  - In older logic families, pulling a line low (to ground) was more reliable and consumed less current than driving it high.

  - Inputs were less sensitive to noise when held low, so designers standardized on active-low signals.

- Simpler Hardware Wiring

  - Many devices (like LEDs) naturally connect with their cathode to ground.

  - Driving the GPIO low completes the circuit → current flows → LED lights up.

  - This avoids needing extra transistors or pull-down resistors.

  Fail-Safe Defaults

  - If a pin is left floating or during reset, it often defaults to high (logic 1).

  - With active-low design, that means the device stays OFF until explicitly enabled.

  - Safer in systems where you don’t want things turning on accidentally.

- Shared Bus / Open-Drain Design

- Active-low signals work well with open-drain/open-collector outputs.

- Multiple devices can pull the line low without conflict.

- Example: I²C bus uses active-low logic for SDA and SCL lines.

  LED connected with cathode to ground, anode to GPIO pin through a resistor.

- GPIO = LOW → current flows → LED ON.

- GPIO = HIGH → no current → LED OFF.

- This wiring is simple, safe, and matches the active-low convention.

11.	BLE

wireless protocol designed for:

- Short-range communication (typically <50m)

- Low power consumption (battery-friendly)

- Sending small amounts of data efficiently

It’s different from classic Bluetooth (used in audio streaming). BLE is ideal for sensors, clocks, smart devices, and wearables.

What is GATT in BLE?

GATT = Generic Attribute Profile

- It defines how data is structured and exchanged between BLE devices.

- GATT sits on top of BLE’s low-level link layer and organizes data into services, characteristics, and descriptors.

- Essentially, GATT is a “data schema” for BLE communication.

- BLE has two main roles:

| Role | Purpose |
| --- | --- |
| Peripheral (Server) | Device that holds the data and advertises it. Example: RTC, heart rate monitor, LED controller. |
| Central (Client) | Device that connects and reads/writes data. Example: smartphone, tablet, ESP32 acting as client |

What is UUID?

UUID = Universally Unique Identifier

- It’s a 128-bit number that uniquely identifies something.

- In BLE, every service and characteristic has a UUID.

- Think of it as a name tag or ID card for your service or characteristic.

  Connect → ESP32 establishes connection to broker (mqtt://broker_ip:1883).

- Subscribe → ESP32 tells broker: “I want messages from test/topic.”

- Publish → ESP32 sends "Hello ESP32" to test/topic.

- Broker forwards → Broker delivers that message to all subscribers of test/topic.

12.	MQTT:

🔹 Step 1: Connect ESP32 to Wi‑Fi

Before MQTT can work, ESP32 must join your router.

This is done with the Wi‑Fi init code:
This ensures ESP32 has an IP address and can reach your broker

🔹 Step 2: Initialize MQTT Client

Once Wi‑Fi is ready, you configure the MQTT client with the broker URI
This is the actual connection step: ESP32 opens a TCP socket to the broker and starts the MQTT session.

🔹 Step 3: Event Handler Confirms Connection

When ESP32 successfully connects, the event handler fires

👉 Notice TCP/IP merges OSI’s 7 layers into 4–5 practical ones.

When ESP32 runs MQTT:

• 	Network Access Layer → Wi‑Fi driver connects to router.

• 	Internet Layer → ESP32 gets an IP address.

• 	Transport Layer → MQTT uses TCP to ensure reliable delivery.

• 	Application Layer → MQTT protocol handles publish/subscribe.

So every MQTT message you send travels down these layers, across the network, then back up the layers on the receiving client.


🔹 Step 1: TCP Connection

- ESP32 knows broker’s IP (192.168.29.53) and port (1883).

- It opens a TCP socket.

- TCP performs the three-way handshake:

  - ESP32 → SYN

  - Broker → SYN-ACK

  - ESP32 → ACK

- ✅ Now a reliable TCP channel exists.

🔹 Step 2: MQTT CONNECT Packet

Once TCP is ready, ESP32 sends an MQTT CONNECT packet. This packet includes:

- Protocol Name & Version → "MQTT" and version (usually 4 for 3.1.1).

- Client ID → unique name for ESP32 (e.g., "ESP32_Client").

- Clean Session flag → tells broker whether to keep old subscriptions.

- Keep Alive timer → how often ESP32 must ping broker (e.g., 60 seconds).

- Username/Password → if broker requires authentication.

🔹 Step 3: Broker Responds (CONNACK)

- Broker checks credentials and session flags.

- Sends back CONNACK packet:

- Return code 0 → connection accepted.

- Non-zero → connection refused (bad credentials, broker unavailable, etc.).

🔹 Step 4: Communication Begins

• 	With TCP ensuring reliable delivery, MQTT now runs smoothly:

• 	Publish → ESP32 sends data to a topic.

• 	Subscribe → ESP32 listens for data on a topic.

• 	Broker → forwards messages to subscribers.

MQTT :

MQTT broker 

Area 1 = temp sensor (client) which provide Message =32. C topic: TempSen

✅ Publisher (send a message)

Open one terminal and run:

mosquitto_pub -h 192.168.29.53 -p 1883 -t test/topic -m "Hello from PC"

- -h → broker host (your PC’s IP, here 192.168.29.53)

- -p → port (1883)

- -t → topic name (test/topic)

- -m → message payload

✅ Subscriber (receive a message)

Open another terminal and run:

mosquitto_sub -h 192.168.29.53 -p 1883 -t test/topic

- This will wait and print any messages published to test/topic.

- If the publisher works, you should see:

Hello from PC